# Literature Review

## 2.1Chapter Overview

This chapter examines the existing literature on the detection of Marine Plastics using remote sensing and machine learning. It begins by providing context on the problem of Marine Plastic Pollution globally and then within the SA EEZ. Thereafter remote sensing approaches applied in previous studies to the detection of marine plastics are described and their shortcomings are noted. SAR is then introduced as a potential alternative and how it is suitable for the detection of marine plastics is discussed. The importance of machine learning coupled with high-quality labelled data is then explored. Finally, research gaps in the consulted literature are identified to justify this study.

## 2.2 Marine Plastic Pollution

### 2.1.1 Classification of Floating Plastics

Plastic is lightweight, durable and inexpensive, making it the material of choice in numerous applications (Nayanathara Thathsarani Pilapitiya and Ratnayake, 2024). While the durability of plastic makes it highly suitable for applications such as packaging, it leads to its long-term persistence in the environment. Polypropylene (PP) and polyethylene (PE) due to their mouldability and lightweight properties are the two most common plastics produced and therefore the most found in oceans as pollutants. These plastics are not biodegradable and therefore can persist for centuries in oceans making them of large environmental concern. The composition of plastics influences whether they are more likely to float on the ocean surface or accumulate on the seafloor. PP and PE are less dense than seawater which allows them to float and disperse on the sea surface, thus may be detected by satellite imagery. Denser plastic polymers such as Polyvinyl Chloride (PVC) and Polyethylene Terephthalate (PET) are more likely to sink accumulating lower in the water column (Thushari and Senevirathna, 2020).

Plastic pollution found floating in marine environments can be classified based on size, ranging from tiny nano-plastics to large macro-plastics. This classification is shown in Table 1 below.

Table 1: Classification of plastic pollution (Thushari and Senevirathna, 2020)

| Plastic Type | Size Range | Description |
| --- | --- | --- |
| Megaplastic | > 1000 mm | Very large items such as buoys and nets. |
| Macroplastic | 20 mm – 1000 mm | Large visible items such as bottles and plastic bags. |
| Mesoplastic | 5 mm – 20mm | Intermediate fragments; degraded macroplastics. |
| Microplastic | < 5mm | Primary Microplastic are small, manufactured plastics such as cosmetic beads. Secondary Microplastics are degraded mesoplastic. |
| Nanoplastic | < 1 µm | Extremely small plastic particles, formed through microplastic fragmentation. |

Plastics floating on the surface do not tend accumulate alone, marine debris is a term that refers to collections of plastic and associated materials such as driftwood and floating vegetation (Danilov and Serdiukova, 2024) the factors driving this accumulation are discussed in Section  2.1.2 below. In this study the focus is primarily on collections of macroplastic, which make up more than 95% of plastic mass in the ocean (Kaandorp et al., 2023), and associated features as marine debris. While microplastics pose significant ecological risks their size and tendency to disperse on the surface make directly monitoring collections with medium resolution satellite imagery unviable.

### 2.1.2 Sources and Aggregation of Marine Plastics

Most plastics that occur in oceans originate from land-based activities from which plastics are transported by rivers, run-off or direct dumping accounting for 70-80% of marine plastic (Thushari and Senevirathna, 2020). Rivers are considered the largest transporter of plastic into oceans, specifically those that run through urban areas before discharging into the ocean (Meijer et al., 2021). Natural disasters such as floods that cause an increase in run off mobilise plastics on the ground and allow for transport into rivers and eventually into oceans (Lincoln et al., 2022). Once plastics reach the ocean from one of these pathways, they have two primary fates namely floating on the ocean surface or sinking to the sea floor where they will ultimately degrade into microplastics (Jamali and Mahdianpari, 2021). It is estimated that only 1% of the plastic present in oceans is floating on the surface while the remaining 99% is on the seafloor or in the sea’s interior (Fauvelle et al., 2021).This is a disheartening statistic as once lost from the surface it is largely more difficult to detect and manage. Plastics that reach the seafloor are more likely to persist for longer as they are no longer affected by wind or UV rays which speed up degradation (Oluwoye et al., 2023). It is therefore important to have strategies in place to detect floating plastics before they are lost from the surface. On the surface plastics are exposed to several ocean surface dynamics that influence their movement and where they accumulate.

Floating material on the ocean surface, including plastics, accumulate where surface flow converges (Nakakuni et al., 2024). Surface flow converges through several different mechanisms at different scales, these include windrows, tidal and frontal convergence and on a large-scale ocean gyres. Windrows refer to long streaks of marine debris that align with wind. These form where under sustained wind Langmuir Cells converge, causing surface downwelling, trapping debris on the surface while forming narrow patterns of debris accumulations that can extend kilometres in length. Plankton also tend to accumulate in windrows making them biologically active zones, increasing the risk of plastic ingestion and entanglement by marine organisms (Aliani et al., 2022). At the boundaries between water masses that differ in salinity, temperature or density, frontal convergence zones are formed which can hold floating marine debris (Shanks, 2021). This may occur where currents meet or near rivers mouths where water fresh water converges with ocean water. Frontal convergence zones form predictable zones of short-term aggregations of floating debris which could point to areas that need immediate observation (van Sebille et al., 2020). Near to the coast dynamics are complex and the movement of floating plastics is dependent on tides, reefs, shelfs and local currents which influence wheter plastics beach, sink or are carried out into open ocean (Shanks, 2021). At a large-scale ocean gyre from large garbage patches in open ocean.  Ocean gyres are large circular systems of ocean currents driven by global wind patterns, forming predictable loops of surface flow accumulating floating plastics (Leal Filho, Hunt and Kovaleva, 2021). There are six major ocean gyres which each have an associated garbage patch, with the Great Pacific Garbage Patch being the largest covering 1.6 million km2 (Leal Filho, Hunt and Kovaleva, 2021). Understanding the processes that drive the accumulation of marine debris are important for identifying areas where plastic is most likely to accumulate and thus where monitoring is most needed.

Windrow study (sediment laden windrows)

Insert images of Windrows as the primary target for evaluation

### 2.1.3 Effects of Plastic in Oceans

General leading statement for the different subcategories of effects.

SDGs Related to plastic pollution (maybe bring in what the DFFE were chatting about with regards to that)

Ecological

Human Health

Economic -> maybe bring in some circular economy

### 2.1.4 South African Context

Research on marine plastic pollution has been an area of focus in South African research of recent (Naidoo, Rajkaran and Sershen, 2020). This has been focused on the effects of plastics on marine ecosystems and the economy from a South African context. South Africa is rich in marine diversity, with a coastline stretching over 2800 km in length. There are 41 Marine Protected Areas (MPAs) which make up 5.4% of the SA EEZ, dedicated to protecting the marine diversity of the country (Field et al., 2025). Marine plastics threaten these environmentally important areas as while human activity is limited, floating plastics can be transported from other areas and settle in MPAs. Marine plastics in South African oceans threaten marine organisms through entanglement and ingestion, the blue economy through reduction in tourism and clean-up costs, as well as human health through the ingestion of plastics accumulated in fish (Arabi and Nahman, 2020).

(Pretorius, Haupt and Sibolla, 2025)

The South African section should emphasise the observation environment—currents, coastal outflows, floods, winds, cloud and likely accumulation zones—more than general biodiversity or GDP statistics.

Section on SA specific marine properties – the currents, the bathymetry etc

Floating plastics emitted from South African land-based sources are expected to be most concentrated near costal urban areas before being transported further out to the Atlantic or Indian Ocean (Ryan, 2020). Locally, this narrows down the areas with the highest need for monitoring to ocean areas near large coastal cities in South Africa such as Durban, Gqeberha and Cape Town. There is little research conducted on the detection and monitoring of marine plastics within South Africa at a large scale, with studies focusing on individual beaches or river systems to sample. Monitoring of the Umgeni river in Durban, which is a major contributor to ocean plastic in the region, found large amounts of floating macro-plastic wate that significantly increased during flooding events (Gutsa et al., 2024). River systems that run through urban areas, such as the Umgeni, serve as pathways for the transport of plastic into oceans which call for monitoring where these rivers meet oceans, specifically after flooding. Remote sensing has the potential to pose a solution to the lack of large-scale plastic monitoring in South Africa with the ability to cover and monitor large areas.

## 2.3 Monitoring Marine Plastic Pollution

### 2.3.1 In-Situ Surveys

In situ methods refer to ground-based surveys of marine plastic where data is collected through direct contact with the environment. These methods allow for accurate monitoring providing quantification of plastics at a local scale. Beach surveys are the simplest way to identify potential sources of marine plastics while estimating the amount of plastic present in the coastal zone from where the litter washes up on (Ryan et al., 2020) while identifying  local marine plastic hotspots (Slimane et al., 2025). Beach surveys have dominated studies on marine plastic monitoring, due to the simplicity and effectiveness on a local scale (Haarr, Falk-Andersson and Fabres, 2022). However, these surveys are largely limited in the area that can be monitored and the time it takes for monitoring to be done.

Research ships can tow trawl nets at the surface to collect floating and subsurface debris for an estimation of plastic concentrations. This method of plastic monitoring is advantageous in that it is able to collect plastics both at and below the sea surface (Ryan et al., 2020). However, ship-based trawling for marine plastic monitoring is limited in where nets can be pulled as well as the high cost involved with fuel and crew in trawl sampling (Pasquier et al., 2022). Instead of trawling nets ships can be fitted with camera systems that continuously capture images of the ocean surface. An object detection algorithm can then be applied to the captured images to recorded floating plastic (de Vries et al., 2021). This has an advantage over trawling in that images can be captured consistently without planned trawls on specialised vessels as well as this method being less invasive to marine life than trawling nets on the surface. However, the same limitations faced in trawling in fuel costs and crew are present in this method of sampling.

Biological monitoring is a form of indirect in-situ monitoring. Marine organisms can serve as bio-indicators of plastic pollution by measuring the amount of ingested plastics (Sacco et al., 2024). While this method can produce insights into pollution levels in the ocean as a whole and the risk of ingestion by humans it does not pin-point the location of plastics for their subsequent removal.

In situ surveys provide ground truth data with high accuracy for understanding marine plastic distribution. However, these methods are labour intensive and largely limited in scalability. It is infeasible for in situ methods to cover large ocean areas, which highlights the need for more scalable monitoring methods such as Remote Sensing.

### 2.3.2 Remote Sensing

Remote Sensing involves using sensors to capture information at the surface of the earth from a distance, without being in contact with the surface (Read and Torrado, 2009). Platforms such as satellites, aircrafts and drones can carry sensors to capture information on a target at different levels of detail and scalability.

Optical remote sensing is a technology that involves the detection and interpretation of electromagnetic energy reflected from the Earth's surface. Sensors are passive meaning that they rely on energy from the sun to hit a target surface before it is reflected and recorded by the sensor. The principle behind using optical imagery for the identification of a feature is because different materials reflect or absorb light in different ways. The pattern of reflection and absorption that a material demonstrates across different wavelengths of light is known as its spectral signature. Optical sensors capture data across different bands of the electromagnetic spectrum allowing for the identification of surface features based on how intense the light is reflected from a surface across different bands.

Certain regions in the electromagnetic spectrum are more sensitive in distinguishing a target feature from a background surface. When applied to detecting marine plastics this would be a region which best separates floating plastics from background ocean water. Waqas et al.(2023) noted that the Red, Near-Infrared (NIR) and Short-Wave Infrared (SWIR) were useful in the detection of marine plastics where there was low absorption by plastics and high absorption by the background ocean water. This was further validated in a study by Knaeps et al., (2021), where the reflective characteristics of marine plastics were measured in a controlled environment.

Spectral indices provide a means of enhancing the detection of a target by using a mathematical combination of bands that separates a specific target material from other background materials. There have been studies that have seen the development of spectral indices specifically to enhance the spectral separation of plastics from background water. Themistocleous et al. (2020) developed the Plastic Index (PI) which makes use of the red and NIR bands for the calculation of the index and was applied to Sentinel-2 Imagery. Biermann et al. (2020) developed the Floating Debris Index (FDI) which too used the red and NIR bands, while including a thresholding technique using SWIR bands to further separate the materials within patches of floating marine debris.

While optical imagery is a well-established data source used to detect marine debris, it faces a large limitation in that it cannot be used in an area with cloud cover, which is particularly prevalent in marine settings (Danilov and Serdiukova, 2024).

Unmanned Autonomous vehicles (UAVs) such as drones have also been used for the detection of marine plastics. Themistocleous et al. (2020) demonstrated the use of UAV high spatial resolution imagery for the detection of plastics in Greece. The classification of plastics was performed on both Sentinel-2 medium-resolution imagery and the UAV high-resolution imagery. It was highlighted that the use of very high-resolution imagery significantly improved the detection and classification accuracy of marine plastics over that of medium spatial resolution of Sentinel-2. Further, Cortesi et al. (2022) presented a study on the detection of river plastic in Italy using a multispectral camera, having the same spectral bands as Sentinel-2, mounted on an aerial UAV, allowing for extremely high spatial resolution. The high spatial resolution allowed the authors to develop an object-based classification, identifying individual pieces of plastic. These results showed very high accuracy in the identification and classification of floating plastics. While these studies have shown high accuracy in detecting floating marine plastics using UAV imagery their practical application to this study is limited. This study aims to develop methods for plastic detection that could be applied to the extent of the SA EEZ, this would not be possible to do with a UAV which offers limited coverage due to maximum flight times. Additionally, UAVs are affected by adverse weather conditions and would not be able to fly during times of heavy winds or rainfall.

## 2.4 Synthetic Aperture Radar (SAR) for Plastic Detection

### 2.4.1 Synthetic Aperture Radar Overview

Synthetic Aperture Radar (SAR) is an active imaging technology that generates images by transmitting microwave signals towards a target area and then capturing the signal scattered back towards the sensor. The return is represented by a complex value containing the strength of the received signal and its phase, which describes the position of the return wave relative to a reference. Because SAR provides its own illumination, it can acquire images during both day and night, unlike optical sensors, which rely on reflected sunlight. SAR operates at microwave wavelengths which are longer than that of visible and near-infrared (NIR) wavelengths measured by optical sensors. Cloud particles strongly scatter or absorb optical wavelengths preventing monitoring over areas with cloud cover. Microwaves interact less with clouds and therefore can travel through cloud cover without image interference. This allows SAR to provide observations under conditions in which optical imagery would be unavailable.

SAR operates in several frequency bands including, increasing in wavelength, Ka-Band, Ku-Band, X-Band, C-Band, S-Band, L-Band and P-Band. The wavelength influences how the radar signal interacts with the target surface. Shorter wavelengths are more sensitive to small scale surface roughness changes and targets close to the surface, while longer wavelengths can penetrate further into materials such a vegetation canopies. The physical properties and scale of the target therefore determine which frequency band is appropriate.

Polarisation refers to the orientation of the transmitted and received signals. SAR systems can be designed to operate in single, dual and quad-polarisation modes. Single polarisation sensors record one polarisation channel, horizontal or vertical, for both transmitting and receiving signals these are denoted as VV or HH for Vertical-Transmit, Vertical-Receive and Horizontal-Transmit, and Horizontal-Receive respectively. Dual polarisation refers to when a sensor transmits in one polarisation either horizontal or vertical and receives in two polarisations, both horizontal and vertical ( NASA-ISRO SAR Mission (NISAR), 2025). The additional cross-polarised channel in dual polarisation imagery provides more information on the target surface allowing for the separation of scattering mechanisms as well as the ability to calculate radar-based indices, such as the polarisation ratio which is the division of the VH value by the VV backscatter value (Mandal et al., 2020). Quad polarisation extends dual polarisation by alternatingly transmitting and receiving in both directions creating all four combinations namely, VV, VH, HV and HH. Quad-pol SAR allows for full polarimetric decomposition, which can separate the dominant form of scattering per pixel (Hajnsek and Desnos, 2021). Decomposition, however, requires phase data to be preserved.

SAR data can be obtained in different product forms, two of the most widely used include Ground Range Detected (GRD) Single Look Complex (SLC) data. SLC data refers to SAR products that are composed of complex valued radar signals meaning each pixel retains information on the amplitude and phase of the returned signal. SLC pixels are represented in a slant-range coordinate system, meaning they retain the full resolution of the sensor (Marghany, 2020). GRD data are SAR products that have been multi-looked and projected to ground range coordinates. Multi-looking refers to the process of averaging neighbouring pixels to improve radiometric resolution while producing square pixels in a regular grid, while this reduces speckle noise in SAR images, it comes at the cost of reducing spatial resolution (Copernicus, 2014). Pixels in GRD images contain only information of the intensity of backscatter, with phase information omitted. Unprocessed Sentinel-1 SAR GRD and SLC scenes are shown in figures 1 & 2 below.

| Figure 1: Unprocessed S1 GRD image. | Figure 2: Unprocessed S1 SLC image. |
| --- | --- |

The choice between SLC and GRD data depends on whether the expected gain from retaining phase information in SLC justifies the additional processing and complexity associated with SLC data. GRD data does not retain phase information meaning it is suitable for applications that only require backscatter intensity values such as general landcover mapping or large object detection. Uniform pixel sizes allows for simpler detection model design as differences in range and azimuth object sizes do not need to be considered (Sørensen, Heiselberg and Heiselberg, 2024). GRD images are more lightweight and have a greater amount of preprocessing applied when compared to SLC data, this makes it preferred for tasks where only backscatter intensity is required, and the resolution lost through multi-looking does not significantly hinder performance. SLC data retains phase, providing more information per pixel, allowing for the derivation of polarimetric information and coherence. The full spatial resolution, while introducing complexity at range views, allows for the detection of finer targets than that in GRD data while using the greater information content, that can be extracted from the retained phase information to separate targets. With regards to plastic detection using SAR, a case for fine not well contrasted target identification, it was found that including SLC-based parameters such as coherence were beneficial in detecting floating objects (Simpson et al., 2022).

Past backscatter intensity, several different features can be extracted from SAR imagery to improve the separation between targets. Texture features consider the spatial arrangement of pixel intensities rather than considering each pixel independently. In the context of marine debris texture features could be used to help identify collections of debris organised by currents and wind based on the difference in texture against the surrounding water. This could otherwise be missed if the average backscatter intensity of the accumulation and surrounding water is similar.

To derive texture features from SAR imagery a Gray Level Co-occurrence Matrix (GLCM) is calculated. Backscatter intensities are grouped into a defined number of grey levels before a sliding window examines how often different combinations of grey levels occur. The GLCM represents the probability of different grey level pairs from occurring within a neighbourhood of pixels. Several features can be calculated from the GCLM such as the GCLM mean, standard deviation and entropy.

Entropy measures the complexity of a local grey level arrangement.

These features allow for the spatial context of objects to be considered which can improve object detection accuracy (Li et al., 2022)

When phase data is retained, as in SLC data, polarimetric features can be derived. Polarimetric decomposition calculates parameters which quantify the dominant form of scattering and the degree of randomness of scattering for an object.

- **Go into Polarimetric Matrix how it can be derived from dual and why we would want to do it, all the features**

(Ferro-Famil and Pottier, 2001)

file:///C:/Users/Joshua%20Pretorius/Documents/Dual-polarimetric_Sentinel-1_SAR_Backscattering_Features_from_Green_Macroalgae_Floating_in_the_Coastal_Ocean.pdf

### 2.4.2 Ocean Surface Physics

- SAR and ocean Scattering mechanisms, from papers on oil and ship detection with SAR.

-In depth on the physics of it (Bragg, Volume, double-bounce)

With Diagram on how it all interacts

### 2.4.2 Floating Plastic and SAR Signal Returns

The principle behind SAR detection of plastics lies in how floating debris alters the roughness of the water surface and thus the received backscatter by the sensor. Floating plastics can both reduce or increase backscatter depending on the type of plastic and the conditions of the surrounding water (de Fockert et al., 2024).Large, dense collections of macroplastics tend to increase surface roughness through creating disturbances in the surrounding water generating brighter SAR returns. Large macroplastic targets have been observed as bright anomalies in contrast to dark, with lower backscatter, surrounding water in SAR imagery (Simpson et al., 2022; Nunziata et al., 2026) . In contrast, microplastics may dampen capillary waves, creating surface slicks causing reduced surface roughness and thus a lower SAR return (Qi et al., 2022). The differences in dielectric constants at the boundary between materials play a role in the signal received by the SAR sensor. The dielectric constant of water is high while that of air is very low, this allows for the clear separation of water and air in SAR imagery. However, the dielectric constant of air and dry plastics are similar, creating a weaker contrast at the plastic-air interface. Therefore, for the separation of plastics and water, surface roughness would be the dominate mechanism influencing the received SAR signal and thus the separation of floating debris and background water (de Fockert et al., 2024).

Wind at the surface of the ocean largely influences surface roughness and thus the backscatter received by the sensor. This can affect the ability to distinguish floating debris induced signal responses from natural variations in sea surface conditions. Under calm conditions, surrounding water is relatively smooth, producing low backscatter. Marine debris accumulations that increase local roughness create a strong contrast against background water making them more likely to appear as bright anomalies in SAR imagery. Under rougher sea surface conditions, this contrast may be reduced. Wind and waves may increase the backscatter from the surrounding water, which can make the background signal more similar to the return from floating debris. As a result, making marine debris collections more difficult to separate from natural surface variability (Nunziata et al., 2026). Therefore to accurately separate the influence of surface conditions on the radar signal from that of floating plastics sea surface variables such as wind speed should be considered and factored into detection methods (Topouzelis, Papakonstantinou and Garaba, 2019). The detection of floating plastics with SAR is thus complex as both dark anomalies, reduced backscatter, and bright anomalies need to be considered as potential litter collections while removing the influence of noise from wind or other unwanted targets. Machine Learning (ML) models can learn these complex patterns in SAR data while accounting for variations in sea surface conditions.

### 2.4.4 Plastic Lookalikes and False Positives in SAR

Where you talk about look alkikes and why the wind data can help us with that insert the reference from kkiaki 2026 they make the recommendation for the sea state variables

Talk about look alike and why the wind, current and SST data can help reduce false positives

- Windrows

- Oil

- Whitecaps/rough sea

- Current lines

## 2.5 Machine Learning

2.5.1 Model Architectures

Machine Learning (ML) is a branch of Artificial Intelligence (AI) which involves the development of algorithms that are able to progressively improve in performance without having to be programmed to do so (Danilov and Serdiukova, 2024). ML algorithms relevant to detection and classification can be broadly grouped into three different types based on the strategy that is used to learn from data; these include supervised learning, unsupervised learning and semi-supervised learning algorithms. Supervised learning algorithms are trained on labelled data with known outcomes and then predict outcomes for new, unknown, data based on what was learnt from the training data. Unsupervised learning algorithms are used to discover patterns within unlabelled data meaning the algorithm is not trained on known training data. Semi-supervised learning combines both labelled and unlabelled data to make predictions on unknown data, combining the supervised and unsupervised approaches (Yang et al., 2023). ML models can be further divided into deep and shallow models based on architecture. Deep learning models are comprised of a number of nonlinear stacked layers that are able to learn complex representations from data while shallow models contain few layers that depend on input features to learn patterns within data (DeLancey et al., 2020). Shallow ML models have been widely applied to Remote Sensing studies, classifying objects on a pixel by pixel basis, including marine debris monitoring however, based on the complexity of SAR data this study would benefit from the inclusion of deep learning models (Janga et al., 2023).

Intro To architecture of deep learning models

Convolutional Neural Networks (CNNs) are a type of deep learning that consider spatial context by classifying objects at a patch level rather than using individual pixels. This patch based approach allows for the model to take the shape of a debris collections into account rather than just the pixel level information (Aires, Boucher and Pellet, 2021).

https://www.researchgate.net/publication/336805909_A_High-Accuracy_Model_Average_Ensemble_of_Convolutional_Neural_Networks_for_Classification_of_Cloud_Image_Patches_on_Small_Datasets/figures?lo=1

Resnet -only patch based classification (no segmentation)

U-net

Visual Transformer

-Good but require huge amounts of data and processing power

While deep learning models have the advantage of considering spatial information as well as understand complex patterns from data they require much more training data than shallow learning models (Taye, 2023). Given the limited availability of training data for marine debris methods such as Self-supervised Learning (SSL) can be used to supplement labelled data allowing the model to learn from unlabelled data before being fine-tuned on the limited labelled datasets.

2.5.2 Training Strategies

SSL involves pertaining models on unlabelled data to learn patterns within the data, through assigning tasks that do not require labelled data such as predicting the rotation of an image or reconstructing a removed region. In the case of detecting floating marine debris with satellite imagery a deep learning model would learn the representations of the ocean surface patterns by solving the tasks in SSL (Li et al., 2024). The model could then be fine-tuned on the smaller labelled dataset to detect marine debris.

Training Strategies

SSL Strategies (MOCO, DINO ect.)

How to train with little true positive data and a high class imbalance (How to make the most out of little data)

So first how to increase volume (Augmentation) and then how to treat the lack of it (so using SSL) and then using methodes for imbalanced datasets (dice ect)

## 2.6 Available Labelled Data for Marine Plastic Detection

To train and fine-tune machine learning models to detect floating marine debris, training data should be used that allows for models to learn and distinguish between floating debris and other targets. This data also allows detections to be validated against true labels. However, collecting in-situ data for marine debris is challenging, due to the dynamic nature of plastic debris. Plastics are influenced by currents, wind drift and waves which means that it is difficult to observe plastics at fixed positions unless the data collected is at the same time of a satellite overpass, which is rarely achievable. To try and overcome this challenge studies have employed other approaches for data creation over traditional in-situ data collection. This includes, manually labelling pixels in satellite images that appear as debris or coincide with reported floating litter, as well as deploying synthetic targets at sea during the time of a satellite overpass (Topouzelis et al., 2020; Kikaki et al., 2022). The training data available for marine plastic debris detection with remote sensing is shown in Table 2 below.

Table 2: Labelled datasets for floating marine plastic detection using remote sensing.

| Dataset | Collection Method | Labels | Sensor | Resolution | Applicability to SAR Detection |
| --- | --- | --- | --- | --- | --- |
| (Savastano et al., 2021) | Used the FDI to identify plastic on S2 Images that had a 4-hour gap between a S1 acquisition. Then manually labelled S1 pixels. | Pixel-wise binary (plastic vs non – Plastic) | Sentinel -1 SLC | 10 m | Used for detection with SAR Imagery – Directly Applicable |
| (Mifdal, Longépé and Rußwurm, 2021) | Manual labelling on S2 Images using NDVI, FDI and visual identification. | Pixel-wise binary (floating vs non-floating objects) | Sentinel -2 | 10 m | Does not separate floating materials. Sentinel-2 images are labelled, could be applicable if temporal gap between acquisitions is low. |
| Plastic Litter Project (PLP) – (Papageorgiou, 2022) | Deployed synthetic targets, plastic bottles and mixed debris according to S2 Pixel sizes. | Pixel-wise binary (plastic vs non – Plastic) | Sentinel -2 and UAV | 10 m | Aimed at optical imagery. However, if targets were out at sea during a S1 overpass could be used to train SAR models. |
| Marine Debris Dataset (MARIDA) – (Kikaki et al., 2022) | Manual labelling on S2 Images using NDVI, FDI and visual identification from experts. | Pixel-wise multi-class (marine debris, wakes, water etc.) | Sentinel -2 | 10 m | Large dataset with multiclass features could be applicable if temporal gap between S1 and S2 is low corresponding to the training dataset. |
| CYGNSS L3 Ocean Microplastic Concentration - (Evans and Ruf, 2022) | Calculated microplastic concentration map based on deviations of expected sea surface roughness at given windspeeds. | Continuous – Microplastic Concentration per km² | CYGNSS (Radar) | 25 km (Grids) | Data is at a coarse scale but could be used for the detection of plastic hotspots detection to identify areas for higher resolution monitoring. |

Add the newer datasets

End of with why this is not enough for SAR and why we need to create a training dataset – which goes into how we will do it

### 2.7 Label Creation

How to create the labels for the data from optical to SAR (has it been done and how can it be done)

Probabilistic drift modelling

How we transfer across from Optical to SAR detections – how we can make it more autonomous through drift modelling

Discuss how Savastano created theirs with MERIA and then the MADO transfer with S2 -> Planet

The label-validity problem. This is arguably the real intellectual centre of your thesis, yet it appears quite late under “Label Creation.” Optical-to-SAR transfer introduces temporal mismatch, movement, drift uncertainty, disagreement between sensors and imperfect visual interpretation. Those labels are proxy labels, not automatically SAR ground truth.

### 2.7 Related Work

There has been a surge in the number of studies aiming to develop methods to accurately identify and classify floating marine plastics. This has seen the development of numerous methods developed based on data from an array of sensors (Waqas et al., 2023). This growth in the field of research points to the importance of being able to monitor plastics effectively.

Kikaki et al.,(2022) introduced  one of the first publicly available, large scale, datasets for marine debris, based on Sentinel-2 imagery. The Marine Debris Archive (MARIDA) contains 837 thousand labelled pixels providing ground truth labels corresponding to 15 different classes of common floating ocean features, including marine debris. The study also set a baseline for the detection of marine debris with optical imagery and machine learning using a U-Net architecture. While focused on Sentinel-2 optical imagery this study sets a precedent for the dataset that will be created in this study. By including co-existing floating features into the dataset, a model can distinguish marine debris from other floating anomalies. This dataset has paved the way for the development of ML models to accurately detect marine debris (Rußwurm, Venkatesa and Tuia, 2023; Alì et al., 2024). In the context of this study the MARIDA data could be used to train models for optical detection to set a baseline to assess the performance of models developed using SAR data. Where there is a corresponding Sentinel-1 acquisition within 24 hours of a Sentinel-2 acquisition SAR data could be extracted from the MARIDA dataset with manual intervention due to the drift of features over the temporal gap.

Booth, Ma and Karakuş, (2023) developed an automated tool for marine debris mapping using Sentinel-2 imagery with deep neural networks. They developed a base model called MAP-Mapper to detect marine debris with optical imagery achieving an overall accuracy of approximately 85%. They produced Marine Debris Map (MDM) index to identify floating plastic densities over time as a form of information dissemination. This work demonstrates the ability of satellite imagery to provide practical information on marine plastic concentrations and hotspots, illustrating best practices for automated plastic monitoring. The reliance on optical imagery, however, highlights its limited applicability to this study which focusses on SAR detection to overcome the limitations of clouds in optical imagery.

The Plastic Litter Project (PLP), is an ongoing project consisting of field experiments with artificial floating plastic targets to quantify their spectral signatures and appearance on Sentinel-2 imagery (Topouzelis, Papakonstantinou and Garaba, 2019). Plastic targets where placed on the ocean and were left over a period of three months to allow for multiple satellite acquisitions of the targets. This allows for the extraction of data past just Sentinel-2, allowing for validated measurements of plastic targets with Sentinel-1 SAR. This data can be used in this study to assess the initial ability of SAR data to detect collections of plastic as well as a proof of concept for placing artificial targets out to sea to create data where floating debris collections are scarce.

Simpson et al. (2022) demonstrated the use of SAR to monitor plastic islands in a river environment. They used change detection algorithms to on Sentinel-1 SLC and GRD data to monitor floating plastic build up achieving high accuracy. It was found that SLC data performed better than GRD data, due to the higher native spatial resolution and the retention of phase information allowing for the calculation of coherence. While this study provides valuable information on the potential of SAR to detect floating plastics, translating methods to a marine environment would involve significant differences to account for the more dynamic nature of the ocean and vaster area.

Qi et al.,(2022) assessed what SAR can detect and what it can’t among floating marine features. This was done by comparing Sentinel-1 and Sentinel-2 observations for different floating features such as macroalgae, cyanobacteria blooms, organic matter and marine debris. It was found that only dense collections of macroalgae consistently appeared as bright patches in SAR images effectively increasing backscatter, while marine debris varied between dark and light patches on Sentinel-1 images. This was reinforced by the findings of de Fockert et al., (2024) in which the backscatter received by a SAR sensor from floating plastic was dependent on the surrounding surface conditions. This research highlights a major limitation of SAR in that the signature of plastics is not consistent and is weak in raw SAR images. Therefore, in this study to detect marine debris with SAR, sophisticated methods that consider the surface conditions and as much information from the SAR image as possible need to be implemented to go beyond what the raw image provides in terms of floating plastic detection.

Savastano et al.,(2021) were the first to apply SAR and AI methods to detect floating marine plastics. A small, labelled SAR dataset was created by using Sentinel-2 detections. The Floating Debris Index (FDI) was applied to detect floating debris collections in Sentinel-2 acquisitions that had a short temporal gap to a corresponding Sentinel-1 acquisition, the collections detected in the Sentinel-2 image were then located in the Sentinel-1 image and were labelled. Shallow machine learning classifiers were then trained on the SAR data to classify floating plastics. Although this approach was preliminary, it demonstrated the feasibility of using SAR to detect floating marine debris and sets the platform for this study to address the limitations of their approach. Small scale shallow models were used; by implementing deep learning in this study more complex patterns could be discovered for more reliable detection. The data created by Savastano et al.,(2021) was approximated from Sentinel-2 detections, by creating a ground truth  SAR-based dataset for marine debris detections could be better validated in SAR images.

Include the study from the SAR returns section

### 2.8 Research Gaps

Through this literature review, there were several research gaps found and recommendations for future work on the methods for the detection of floating marine plastics. There is a large reliance on the use of optical imagery, such as Sentinel-2, to detect floating marine debris. This has led to the development of large datasets and base models  for accurate monitoring and detection (Waqas et al., 2023). However, optical imagery faces a major limitation in that monitoring cannot take place when there is cloud cover, which is often the case during and after events that release large amounts of plastic into the ocean such as during floods. To overcome this limitation other data sources that operate regardless of cloud cover need to be investigated, such as SAR. The use of SAR for the detection of marine debris is largely unexplored and is still in research infancy, posing as a major gap in the literature on marine plastic detection (Savastano et al., 2021). With little research on SAR-based methods there are few existing training datasets and base models that would allow for the development of practical SAR-based detection methodologies to overcome the limitation of cloud cover in optical imagery moving toward more consistent plastic monitoring. Aside from the work by Savastano et al.,( 2021) using shallow classifiers for a binary classification, there have been no published deep learning models for plastic detection with SAR imagery. This could overcome the challenges faced in trying to visually identify marine debris in SAR data highlighted by Qi et al.,(2022), by learning complex patterns in SAR signals and interactions with surface conditions. With limited training data for marine debris detection with SAR imagery, the creation of a dataset would not only allow for models to be trained in this study but would address the gap in the literature, allowing for future studies to build on the dataset and improve model performance. Little research on marine debris detection has been conducted in the SA EEZ and Africa as a whole, this research will aim to address this gap by implementing a monitoring method suited for the region while creating a dataset  within SA EEZ (Naidoo, Rajkaran and Sershen, 2020).

Aires, F., Boucher, E. and Pellet, V. (2021) “Convolutional neural networks for satellite remote sensing at coarse resolution. Application for the SST retrieval using IASI,” Remote Sensing of Environment, 263, p. 112553. Available at: https://doi.org/10.1016/j.rse.2021.112553.

Alì, M. et al. (2024) “Assessment of Spectral based Solutions for the Detection of Floating Marine Debris.” arXiv. Available at: https://doi.org/10.48550/arXiv.2408.10187.

Aliani, S. et al. (2022) “Editorial: Marine Litter Windrows,” Frontiers in Marine Science, 8. Available at: https://doi.org/10.3389/fmars.2021.827907.

Arabi, S. and Nahman, A. (2020) “Impacts of marine plastic on ecosystem services and economy: State of South African research,” South African Journal of Science, 116(5/6). Available at: https://doi.org/10.17159/sajs.2020/7695.

Biermann, L. et al. (2020) “Finding Plastic Patches in Coastal Waters using Optical Satellite Data,” Scientific Reports, 10(1), p. 5364. Available at: https://doi.org/10.1038/s41598-020-62298-z.

Booth, H., Ma, W. and Karakuş, O. (2023) “High-precision density mapping of marine debris and floating plastics via satellite imagery,” Scientific Reports, 13(1), p. 6822. Available at: https://doi.org/10.1038/s41598-023-33612-2.

Copernicus (2014) “Sentinel-1 – Documentation. Copernicus Data Space.” Available at: https://documentation.dataspace.copernicus.eu/Data/SentinelMissions/Sentinel1.html (Accessed: October 14, 2025).

Danilov, A. and Serdiukova, E. (2024) “Review of Methods for Automatic Plastic Detection in Water Areas Using Satellite Images and Machine Learning,” Sensors (Basel, Switzerland), 24(16), p. 5089. Available at: https://doi.org/10.3390/s24165089.

DeLancey, E.R. et al. (2020) “Comparing Deep Learning and Shallow Learning for Large-Scale Wetland Classification in Alberta, Canada,” Remote Sensing, 12(1), p. 2. Available at: https://doi.org/10.3390/rs12010002.

Evans, M.C. and Ruf, C.S. (2022) “Toward the Detection and Imaging of Ocean Microplastics With a Spaceborne Radar,” IEEE Transactions on Geoscience and Remote Sensing, 60, pp. 1–9. Available at: https://doi.org/10.1109/TGRS.2021.3081691.

Fauvelle, V. et al. (2021) “Organic additive release from plastic to seawater is lower under deep-sea conditions,” Nature Communications, 12(1), p. 4426. Available at: https://doi.org/10.1038/s41467-021-24738-w.

Ferro-Famil, L. and Pottier, E. (2001) “Dual Frequency Polarimetric SAR Data Classification and Analysis,” Progress In Electromagnetics Research, 31, pp. 247–272. Available at: https://doi.org/10.2528/PIER00081601.

Field, L.C. et al. (2025) “Assessing South Africa’s marine protected area quality and progress towards conservation goals: An application of The MPA Guide framework,” Marine Policy, 173, p. 106513. Available at: https://doi.org/10.1016/j.marpol.2024.106513.

de Fockert, A. et al. (2024) “Assessing the detection of floating plastic litter with advanced remote sensing technologies in a hydrodynamic test facility,” Scientific Reports, 14(1), p. 25902. Available at: https://doi.org/10.1038/s41598-024-74332-5.

Gutsa, T. et al. (2024) “Wasted shores: Using drones to monitor the spatio-temporal evolution of debris accumulation hotspots on South Africa’s Umgeni River,” Science of The Total Environment, 955, p. 176791. Available at: https://doi.org/10.1016/j.scitotenv.2024.176791.

Haarr, M.L., Falk-Andersson, J. and Fabres, J. (2022) “Global marine litter research 2015–2020: Geographical and methodological trends,” Science of The Total Environment, 820, p. 153162. Available at: https://doi.org/10.1016/j.scitotenv.2022.153162.

Hajnsek, I. and Desnos, Y.-L. (2021) Polarimetric Synthetic Aperture Radar Principles and Application: Principles and Application. Available at: https://doi.org/10.1007/978-3-030-56504-6.

Jamali, A. and Mahdianpari, M. (2021) “A Cloud-Based Framework for Large-Scale Monitoring of Ocean Plastics Using Multi-Spectral Satellite Imagery and Generative Adversarial Network,” Water, 13(18), p. 2553. Available at: https://doi.org/10.3390/w13182553.

Janga, B. et al. (2023) “A Review of Practical AI for Remote Sensing in Earth Sciences,” Remote Sensing, 15(16), p. 4112. Available at: https://doi.org/10.3390/rs15164112.

Kaandorp, M.L.A. et al. (2023) “Global mass of buoyant marine plastics dominated by large long-lived debris,” Nature Geoscience, 16(8), pp. 689–694. Available at: https://doi.org/10.1038/s41561-023-01216-0.

Kikaki, K. et al. (2022) “MARIDA: A benchmark for Marine Debris detection from Sentinel-2 remote sensing data,” PLOS ONE, 17(1), p. e0262247. Available at: https://doi.org/10.1371/journal.pone.0262247.

Knaeps, E. et al. (2021) “Hyperspectral-reflectance dataset of dry, wet and submerged marine litter,” Earth System Science Data, 13(2), pp. 713–730. Available at: https://doi.org/10.5194/essd-13-713-2021.

Leal Filho, W., Hunt, J. and Kovaleva, M. (2021) “Garbage Patches and Their Environmental Implications in a Plastisphere,” Journal of Marine Science and Engineering, 9(11), p. 1289. Available at: https://doi.org/10.3390/jmse9111289.

Li, K. et al. (2022) “Detection of oil spills based on gray level co-occurrence matrix and support vector machine,” Frontiers in Environmental Science, 10. Available at: https://doi.org/10.3389/fenvs.2022.1049880.

Li, W. et al. (2024) “Predicting gradient is better: Exploring self-supervised learning for SAR ATR with a joint-embedding predictive architecture,” ISPRS Journal of Photogrammetry and Remote Sensing, 218, pp. 326–338. Available at: https://doi.org/10.1016/j.isprsjprs.2024.09.013.

Lincoln, S. et al. (2022) “Marine litter and climate change: Inextricably connected threats to the world’s oceans,” Science of The Total Environment, 837, p. 155709. Available at: https://doi.org/10.1016/j.scitotenv.2022.155709.

Mandal, D. et al. (2020) “Dual polarimetric radar vegetation index for crop growth monitoring using sentinel-1 SAR data,” Remote Sensing of Environment, 247, p. 111954. Available at: https://doi.org/10.1016/j.rse.2020.111954.

Marghany, M. (2020) “Chapter 8 - Principle theories of synthetic aperture radar,” in M. Marghany (ed.) Synthetic Aperture Radar Imaging Mechanism for Oil Spills. Gulf Professional Publishing, pp. 127–150. Available at: https://doi.org/10.1016/B978-0-12-818111-9.00008-2.

Meijer, L.J.J. et al. (2021) “More than 1000 rivers account for 80% of global riverine plastic emissions into the ocean,” Science Advances, 7(18), p. eaaz5803. Available at: https://doi.org/10.1126/sciadv.aaz5803.

Mifdal, J., Longépé, N. and Rußwurm, M. (2021) “TOWARDS DETECTING FLOATING OBJECTS ON A GLOBAL SCALE WITH LEARNED SPATIAL FEATURES USING SENTINEL 2,” ISPRS Annals of the Photogrammetry, Remote Sensing and Spatial Information Sciences, V-3–2021, pp. 285–293. Available at: https://doi.org/10.5194/isprs-annals-V-3-2021-285-2021.

Naidoo, T., Rajkaran, A. and Sershen (2020) “Impacts of plastic debris on biota and implications for human health: A South African perspective,” South African Journal of Science, 116(5/6). Available at: https://doi.org/10.17159/sajs.2020/7693.

Nakakuni, M. et al. (2024) “Convergence zones of coastal waters as hotspots for floating microplastic accumulation,” Marine Pollution Bulletin, 206, p. 116691. Available at: https://doi.org/10.1016/j.marpolbul.2024.116691.

Nayanathara Thathsarani Pilapitiya, P.G.C. and Ratnayake, A.S. (2024) “The world of plastic waste: A review,” Cleaner Materials, 11, p. 100220. Available at: https://doi.org/10.1016/j.clema.2024.100220.

Nunziata, F. et al. (2026) “Detecting Coastal Aggregation of Macroplastic Litter Using Satellite X-Band SAR Imagery,” IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, 19, pp. 10794–10803. Available at: https://doi.org/10.1109/JSTARS.2026.3675344.

Oluwoye, I. et al. (2023) “Degradation and lifetime prediction of plastics in subsea and offshore infrastructures,” Science of The Total Environment, 904, p. 166719. Available at: https://doi.org/10.1016/j.scitotenv.2023.166719.

Pasquier, G. et al. (2022) “An innovative approach for microplastic sampling in all surface water bodies using an aquatic drone,” Heliyon, 8(11), p. e11662. Available at: https://doi.org/10.1016/j.heliyon.2022.e11662.

Polarimetry | Get to Know SAR – NASA-ISRO SAR Mission (NISAR) (no date). Available at: https://nisar.jpl.nasa.gov/mission/get-to-know-sar/polarimetry/ (Accessed: June 25, 2025).

Pretorius, J., Haupt, S. and Sibolla, B. (2025) “Exploring the potential of remote sensing to detect marine plastic debris in the South African Ocean region,” ISPRS Annals of the Photogrammetry, Remote Sensing and Spatial Information Sciences, X-G-2025, pp. 665–671. Available at: https://doi.org/10.5194/isprs-annals-X-G-2025-665-2025.

Qi, L. et al. (2022) “On the capacity of Sentinel-1 synthetic aperture radar in detecting floating macroalgae and other floating matters,” Remote Sensing of Environment, 280, p. 113188. Available at: https://doi.org/10.1016/j.rse.2022.113188.

Read, J.M. and Torrado, M. (2009) “Remote Sensing,” in R. Kitchin and N. Thrift (eds.) International Encyclopedia of Human Geography. Oxford: Elsevier, pp. 335–346. Available at: https://doi.org/10.1016/B978-008044910-4.00508-3.

Rußwurm, M., Venkatesa, S.J. and Tuia, D. (2023) “Large-scale detection of marine debris in coastal areas with Sentinel-2,” iScience, 26(12), p. 108402. Available at: https://doi.org/10.1016/j.isci.2023.108402.

Ryan, P.G. et al. (2020) “Monitoring marine plastics - will we know if we are making a difference?,” South African Journal of Science, 116(5–6), pp. 1–9. Available at: https://doi.org/10.17159/sajs.2020/7678.

Ryan, P.G. (2020) “The transport and fate of marine plastics in South Africa and adjacent oceans,” South African Journal of Science, 116(5/6). Available at: https://doi.org/10.17159/sajs.2020/7677.

Sacco, V.A. et al. (2024) “What are the global patterns of microplastic ingestion by fish? A scientometric review,” Environmental Pollution, 350, p. 123972. Available at: https://doi.org/10.1016/j.envpol.2024.123972.

Savastano, S. et al. (2021a) “A First Approach to the Automatic Detection of Marine Litter in SAR Images Using Artificial Intelligence,” 2021 IEEE International Geoscience and Remote Sensing Symposium IGARSS. 2021 IEEE International Geoscience and Remote Sensing Symposium IGARSS, pp. 8704–8707. Available at: https://doi.org/10.1109/IGARSS47720.2021.9737038.

Savastano, S. et al. (2021b) “A First Approach to the Automatic Detection of Marine Litter in SAR Images Using Artificial Intelligence,” 2021 IEEE International Geoscience and Remote Sensing Symposium IGARSS. 2021 IEEE International Geoscience and Remote Sensing Symposium IGARSS, pp. 8704–8707. Available at: https://doi.org/10.1109/IGARSS47720.2021.9737038.

van Sebille, E. et al. (2020) “The physical oceanography of the transport of floating marine debris,” Environmental Research Letters, 15(2), p. 023003. Available at: https://doi.org/10.1088/1748-9326/ab6d7d.

Shanks, A.L. (2021) “Observational Evidence and Open Questions on the Role of Internal Tidal Waves on the Concentration and Transport of Floating Plastic Debris,” Frontiers in Marine Science, 8. Available at: https://doi.org/10.3389/fmars.2021.621062.

Simpson, M.D. et al. (2022) “Monitoring of Plastic Islands in River Environment Using Sentinel-1 SAR Data,” Remote Sensing, 14(18), p. 4473. Available at: https://doi.org/10.3390/rs14184473.

Slimane, E.B. et al. (2025) “Efficient Beach Litter Monitoring: Accelerated Surveys of Pollution Hotspots—A North African Case Study,” Journal of Marine Science and Engineering, 13(1), p. 71. Available at: https://doi.org/10.3390/jmse13010071.

Sørensen, K.A., Heiselberg, P. and Heiselberg, H. (2024) “Unified Detection and Feature Extraction of Ships in Satellite Images,” Remote Sensing, 16(24). Available at: https://doi.org/10.3390/rs16244719.

Taye, M. (2023) “Understanding of Machine Learning with Deep Learning: Architectures, Workflow, Applications and Future Directions,” Computers [Preprint]. Available at: https://doi.org/10.3390/COMPUTERS12050091.

Themistocleous, K. et al. (2020) “Investigating Detection of Floating Plastic Litter from Space Using Sentinel-2 Imagery,” Remote Sensing, 12(16), p. 2648. Available at: https://doi.org/10.3390/rs12162648.

Thushari, G.G.N. and Senevirathna, J.D.M. (2020) “Plastic pollution in the marine environment,” Heliyon, 6(8), p. e04709. Available at: https://doi.org/10.1016/j.heliyon.2020.e04709.

Topouzelis, K. et al. (2020) “Remote Sensing of Sea Surface Artificial Floating Plastic Targets with Sentinel-2 and Unmanned Aerial Systems (Plastic Litter Project 2019),” Remote Sensing, 12(12), p. 2013. Available at: https://doi.org/10.3390/rs12122013.

Topouzelis, K., Papakonstantinou, A. and Garaba, S.P. (2019) “Detection of floating plastics from satellite and unmanned aerial systems (Plastic Litter Project 2018),” International Journal of Applied Earth Observation and Geoinformation, 79, pp. 175–183. Available at: https://doi.org/10.1016/j.jag.2019.03.011.

de Vries, R. et al. (2021) “Quantifying Floating Plastic Debris at Sea Using Vessel-Based Optical Data and Artificial Intelligence,” Remote Sensing, 13(17), p. 3401. Available at: https://doi.org/10.3390/rs13173401.

Waqas, M. et al. (2023) “Marine plastic pollution detection and identification by using remote sensing-meta analysis,” Marine Pollution Bulletin, 197, p. 115746. Available at: https://doi.org/10.1016/j.marpolbul.2023.115746.

Yang, X. et al. (2023) “A Survey on Deep Semi-supervised Learning,” IEEE Transactions on Knowledge and Data Engineering, 35(9), pp. 8934–8954. Available at: https://doi.org/10.1109/TKDE.2022.3220219.